What is Digital Fundamentals-

Digital Fundamentals is a field within electronics and computer engineering that focuses on systems that handle information in the form of binary digits i.e.0 &1. These systems operate using just two logic states, 0 and 1, which makes them very reliable and efficient for modern computing tasks.

Unlike analog systems, which process information through continuous values, digital systems use discrete values.

Because of their accuracy, speed, and ease of implementation, digital techniques are commonly used in today's electronic devices and communication systems.

Some Advantages-

Common Uses of Digital Systems Personal Computers and Laptops Mobile Phones and Smartphones Digital Calculators

Smart Watches and Electronic Clocks Automated Teller Machines (ATMs) Traffic Monitoring and Control Systems

Analog and Digital Signals

Analog Signals

An analog signal changes continuously over time and can take on an infinite number of values within a given range.

These signals are often found in natural events.

Examples Human speech

Changes in atmospheric temperature Audio signals and sound waves

Digital Signals

A digital signal uses fixed voltage levels to represent information as binary values 0 and 1. Since there are only two states, digital signals are easier to process and store.

Benefits of Digital Signals

- More accurate data processing.

- Better resistance to noise and interference.

- Easier storage and retrieval of information Reliable communication and data transmission.

Comparison Between Analog and Digital Systems Analog Systems Digital Systems

Information varies continuously Information is in discrete levels More sensitive to noise Stronger resistance to noise

Difficult to store Quick and efficient storage Lower reliability Higher reliability

What is Number Systems

A number system is a way to represent numerical values using a set of symbols. Different number systems are used in digital electronics to carry out various operations.

- Decimal Number System (Base 10)

The decimal system uses ten symbols, from 0 to 9.

Example: (264)₁₀

- Binary Number System (Base 2)

The binary system uses only two digits: 0 and 1. It is the basic language of digital electronics.

Example: (100101)₂

- Octal Number System (Base 8)

The octal system uses digits from 0 to 7.

Example: (267)₈

- Hexadecimal Number System (Base 16)

This system uses ten digits and six letters (A–F).

Example: (2AF)₁₆

Binary Arithmetic

Binary arithmetic is similar to decimal arithmetic but uses only two digits.

Rules for Binary Addition

A        B          Sum Carry

0         0             0        0

0         1             1        0

1         0             1        0

1         1             0        1

Example:

1011₂ + 0101₂ = 10000₂

Binary Subtraction Using Two's Complement

Two's complement is used to subtract numbers in digital systems.

Procedure

- Find the one's complement of the subtrahend. 2. Add 1 to get the two's complement.

- Add it to the minuend.

- If there is an extra bit i.e. Carry discard it and also put positive sign in it.

- If there is no carry then put negetive sign with 2’s complement of the result.

Signed and Unsigned Number Representation

Unsigned Numbers-

Unsigned numbers represent only non-negative values. Example for a 4-bit number:

0000 = 0

1111 = 15

Signed Numbers

In signed representation, the most significant bit indicates the sign of the number.

MSB = 0 → Positive Number MSB = 1 → Negative Number

Examples:

0101 = +5

1101 = −3 (using two's complement) Logic Gates

Logic gates are electronic circuits that perform basic logical operations on binary inputs.

AND Gate

The output is 1 only when all inputs are 1.

Expression:

Y = A · B

OR Gate

The output is 1 when at least one input is 1.

Expression:

Y = A + B

NOT Gate

The NOT gate reverses the input state.

Expression:

Y = A'

NAND Gate

A NAND gate is the output of an AND gate with an inverted result. It is considered a universal gate.

NOR Gate

A NOR gate is the output of an OR gate with an inverted result.

It is also a universal gate. XOR Gate

The XOR gate gives a high output when the inputs are different.

Expression:

Y = A ⊕ B

XNOR Gate

The XNOR gate provides a high output when the inputs are the same.

Expression:

Y = A ⊙ B

Boolean Algebra

Boolean Algebra offers mathematical methods for analyzing and simplifying logical expressions.

Basic Laws

A + 0 = A A · 1 = A A + 1 = 1

A · 0 = 0 A + A = A A · A = A A + A' = 1 A · A' = 0

De Morgan's Theorems

These theorems are used to simplify logic circuits.

(A + B)' = A'B'

(AB)' = A' + B'

Karnaugh Maps (K-Maps)

A Karnaugh Map is a visual method used to simplify Boolean expressions without heavy algebra.

Types

Two-variable K-Map Three-variable K-Map Four-variable K-Map Advantages

Simplifies logical expressions Reduces hardware needs Minimizes gate count Improves circuit efficiency Guidelines

Group adjacent cells with logic 1.

Valid groups include 1, 2, 4, 8, or 16 cells. Form the largest possible groups.

Combinational Logic Circuits

In combinational circuits, the output depends entirely on the current input.

Half Adder

Adds two binary digits.

Output equations:

Sum = A ⊕ B Carry = AB Full Adder

Adds three binary inputs, including a carry input.

Inputs:

A B

Carry-in (Cin) Outputs:

Sum Carry-out

Half Subtractor

Produces difference and borrow for two-bit subtraction.

Full Subtractor

Extends subtraction by adding a borrow input.

Multiplexer (MUX)

A multiplexer selects one input from several options and transfers it to the output.

Decoder

A decoder changes n binary inputs into 2ⁿ unique outputs.

Encoder

An encoder does the opposite by converting multiple inputs into a binary code.

Sequential Logic Circuits

Unlike combinational circuits, sequential circuits depend on both current inputs and stored information.

Flip-Flops

A flip-flop is a memory element that stores a single binary digit.

SR Flip-Flop

Offers set and reset operations.

JK Flip-Flop

Eliminates the invalid state found in SR flip-flops.

D Flip-Flop

Used in memory devices and registers.

T Flip-Flop

Used in counting applications.

Registers

Registers used to store data consisting multiple flip-flop to store & transfer binary data temporary.flip-flop stores one bit at a time , so if we want to store an n-bit register then we need of n flip-flops.

Features:

- It is used to Stores binary data .

- It is Used for temporary storing in digital systems.

- It can perform 4 operations 1. load  2. clear  3. shift   4. transfer.

Types:

There are multiple types of Registers.

- Serial-In Serial-Out (SISO) – Data enters serially & exits one bit at a time.

- Serial-In Parallel-Out (SIPO) – Data enters serially & read in parallel.

- Parallel-In Serial-Out (PISO) – Data is loaded in parallel and shifted out serially.

- Parallel-In Parallel-Out (PIPO) – Data is loaded and read in parallel.

- Shift Register – Shifting data from left or right.

Counters

It is a sequential digital circuit that counts clock pulses and made using flip-flops. It changes its state with each clock pulse.

Features:

- Counts clock pulses & events.

- It Produce a sequence of binary numbers.

- It is Used in application like timing and control.

Types of Counters:

- Asynchronous (Ripple) Counter

  - Flip-flops are triggered one after another.

  - It is slower due to delay(propagation).

  - The design is very Simple.

- Synchronous Counter

  - All flip-flops receive same clock signal.

  - It is Faster and more reliable than Asynchronous counter.

  - It having Preference in modern digital systems.

- Up Counter

  - Counting in ascending order (0, 1, 2, 3...).

- Down Counter

  - Counting in descending order (3, 2, 1, 0...).

- Up/Down Counter

  - Can count in both order upward as well as downward.