Sequential Logic Circuits

Unlike combinational circuits, sequential circuits depend on both current inputs and stored information.

Flip-Flops

A flip-flop is a memory element that stores a single binary digit.

SR Flip-Flop

Offers set and reset operations.

JK Flip-Flop

Eliminates the invalid state found in SR flip-flops.

D Flip-Flop

Used in memory devices and registers.

T Flip-Flop

Used in counting applications.

Registers

Registers consist of multiple flip-flops that store and transfer binary data temporarily. Types of Registers - 1. Shift Registers

These registers not only store data but also allow data bits to shift either left or right upon receiving a clock pulse. They are primarily used for data transfer and manipulation. [1, 2]

They are divided into four main types depending on how data enters and exits the register: [1, 2]

- SISO (Serial-In, Serial-Out): Data is entered one bit at a time (serially) and retrieved one bit at a time. It acts as a delay line for data streams. [1, 2, 3, 4]

- SIPO (Serial-In, Parallel-Out): Data enters one bit at a time serially, but all bits are extracted simultaneously in parallel. It is often used as a serial-to-parallel converter. [1]

- PISO (Parallel-In, Serial-Out): Data is loaded into the register simultaneously (in parallel), but the bits are shifted and outputted one by one (serially). It is used for parallel-to-serial conversion. [1, 2]

- PIPO (Parallel-In, Parallel-Out): Data is loaded in parallel and outputted in parallel. It is the fastest shift register, as data is available immediately after one clock pulse. [1, 2, 3]

- Bidirectional Shift Register: Allows data to be shifted both left and right via a control signal. [1, 2, 3]

- Universal Shift Register: Combines all functions (SISO, SIPO, PISO, PIPO) in one circuit, allowing both serial and parallel operations. [1, 2]

2. Storage / Buffer Registers

These registers are primarily used to temporarily hold binary data (information or instructions) that is being transferred between the CPU and input/output devices. [1, 2]

- Buffer Register: Simply holds a block of data during transfer. The bits are loaded and retrieved simultaneously.

3. CPU (Microprocessor) Registers

Within a central processing unit (CPU), specific registers perform targeted tasks to manage program execution: [1, 2]

- Memory Address Register (MAR): Stores the memory address of the data or instruction currently being fetched or written.

- Memory Data Register (MDR) / Memory Buffer Register: Temporarily stores the actual data or instructions being fetched from or sent to memory.

- Program Counter (PC): Keeps track of the memory address of the next instruction to be executed.

- Instruction Register (IR): Holds the instruction currently being executed by the CPU.

- Accumulator: A general-purpose register that holds intermediate results of arithmetic and logical operations.

- General-Purpose Registers: Used by the CPU to store variables and temporary data during execution (e.g., AX, BX, CX in x86 architecture).

- Status / Flag Register: Monitors the status of the CPU and operations (e.g., indicating zero, carry, or overflow flags after a calculation)

Counters - A counter in digital electronics is a sequential logic circuit used to count the number of clock pulses or events applied to its input. Composed of cascaded flip-flops, it advances through a predetermined sequence of states, performing tasks like frequency division, time tracking, and event counting. [1, 2, 3, 4]

Counters are primarily categorized by how the clock signal is applied and the sequence they follow. [1, 2]

1. Classification by Clocking Mechanism

- Asynchronous (Ripple) Counters: The primary clock pulse is applied only to the first flip-flop. The output of each stage provides the clock for the next stage. This cascading "ripple" effect causes propagation delays, making them slower but simpler to design. [1, 2, 3]

- Synchronous Counters: All flip-flops are triggered simultaneously by the same global clock signal. Because they all change state at the same time, they are faster and eliminate decoding errors, but require more complex combinatorial logic. [1, 2]

2. Classification by Counting Sequence

- Up Counters: Count incrementally from 0 to a maximum value.

- Down Counters: Count decrementally from a maximum value down to 0.

- Up/Down Counters: Can count in both directions, typically controlled by an external input/mode pin.

- Mod-N Counters: Truncated counters that recycle after a specific number of states. For example, a Mod-10 (Decade) counter counts from 0 to 9 (10 states total) and then resets to 0. [1, 2, 3, 4]

3. Classification by Operation

- Binary Counters: Count in straight binary sequences (e.g., 000, 001, 010, 011). An n-bit binary counter has \(2^{n}\) states and can count up to \(2^n - 1\). [1, 2, 3, 4, 5]

- BCD (Decimal) Counters: Count in Binary Coded Decimal, expressing each decimal digit (0-9) with 4 bits. [1]

- Specialized Counters: Includes Ring Counters (where the output of the last shift register loops back to the first) and Johnson Counters (a modified ring counter with an inverted feedback). [1, 2, 3, 4, 5]

Key Design Principles

The number of flip-flops (n) required to build a counter is based on the number of states (N) being represented. The relationship is governed by the inequality:

For example, to design a mod-5 counter, you need 3 flip-flops because 2² = 4 is too small, but 2³ = 8 provides enough states. [1]

Common Applications

- Digital Clocks and Watches: Tracking seconds, minutes, and hours using frequency division.

- Frequency Dividers: Dividing a high-frequency clock signal into a lower, usable frequency.

- Timers: Creating precise time delays for microcontrollers and processors.

- Event Counting: Industrial measurement modules used in manufacturing for counting products on a conveyor line